VIETNAMESE-GERMAN UNIVERSITY

Programming Methods 2  |  Mechatronics  |  Instructor: Bien Minh Tri

MuJoCo Car Simulation

Mathematical Formulas & Navigation Algorithms

Rule-Based Obstacle Avoidance (car_controller.py)

Group Members:

Nguyen Huu Thuc   —   11525034

Tran Duc Minh   —   11525063

Nguyen Canh Nhat Nguyen   —   11525051

# 1. Car State & Coordinate System

The car operates in a 2D world frame (XY plane). Its full state at each timestep is:

State = (x,  y,  yaw)

x, y  : world-frame position (meters)

yaw   : heading angle from +X axis (radians, range [-pi, pi])

## 1.1  Reading State from MuJoCo Sensors

Position is read from the 'car_pos' framepos sensor, heading from 'car_xaxis':

pos = sensor('car_pos').data[:2]

yaw = atan2( xaxis.y,  xaxis.x )

## 1.2  Angle Wrapping

All angle differences are normalized to [-pi, pi] to prevent discontinuities:

wrap(a)  =  ((a + pi) mod 2*pi)  -  pi

# 2. Velocity Control

The car is driven by two control signals: linear velocity v (m/s) and angular velocity omega (rad/s). These are written directly to the freejoint degrees of freedom:

## 2.1  World-Frame Projection

vx = v * cos(yaw)

vy = v * sin(yaw)

Local forward velocity v is projected onto world X and Y axes.

## 2.2  Joint Velocity Assignment  (set_vel)

qvel[qa+0] = v * cos(yaw)    # slide X

qvel[qa+1] = v * sin(yaw)    # slide Y

qvel[qa+2] = qvel[qa+3] = qvel[qa+4] = 0

qvel[qa+5] = omega            # hinge Z (yaw rate)

## 2.3  Physics Integration (RK4, dt = 0.01 s)

x(t+dt)   =  x(t)   +  vx * dt

y(t+dt)   =  y(t)   +  vy * dt

yaw(t+dt) =  yaw(t) +  omega * dt

# 3. Goal Navigation

At every timestep, the controller computes direction and distance to the goal, then derives a heading correction.

## 3.1  Distance to Goal

to_goal   =  goal - pos  =  [gx - x,   gy - y]

d_goal    =  ||to_goal|| =  sqrt((gx-x)^2 + (gy-y)^2)

## 3.2  Goal Angle & Heading Error

goal_angle  =  atan2(gy - y,   gx - x)

herr        =  wrap( goal_angle - yaw )

herr > 0  ->  goal is to the LEFT  (steer left, omega > 0)

herr < 0  ->  goal is to the RIGHT (steer right, omega < 0)

herr = 0  ->  car is pointing directly at goal

## 3.3  Proportional Heading Correction

omega  =  clip( Kp * herr,  -omega_max,  +omega_max )

Kp        = 2.2      (proportional gain, from code: 2.2*herr)

omega_max = 1.6  rad/s

## 3.4  Goal Arrival Condition

ARRIVED   if   d_goal  <  goal_tol  (= 0.55 m)

# 4. Virtual Ray-Cast Sensor  (RayCastSensor)

Three virtual rays are emitted analytically from the car front. No physics sensor plugin is used. Each ray checks all obstacle AABBs using the slab method.

## 4.1  Ray Angles

ray_center = yaw  +  0

ray_left   = yaw  +  FOV      (FOV = 0.55 rad  ~  31.5 deg)

ray_right  = yaw  -  FOV

Ray direction vector:  d = [cos(angle), sin(angle)]

## 4.2  Ray-AABB Intersection  (Slab Method — _hit function)

For each obstacle bounding box centered at (ox, oy) with half-size hs:

txlo = (ox - hs - px) / dx

txhi = (ox + hs - px) / dx

tylo = (oy - hs - py) / dy

tyhi = (oy + hs - py) / dy

t0 = max( min(txlo,txhi),  min(tylo,tyhi) )

t1 = min( max(txlo,txhi),  max(tylo,tyhi) )

px, py : ray origin (car position)

dx, dy : ray direction components  (with epsilon=1e-12 guard)

ox, oy : obstacle center;  hs : obstacle half-size

## 4.3  Hit Condition & Distance

HIT  if  t1 >= 0  AND  t0 <= t1  AND  t0 <= sense_range

dist = max(0,  t0)

sense_range = 2.0 m  (no hit -> dist = 2.0)

## 4.4  Scan Output

(dc, dl, dr) = sensor.scan(pos, yaw)

dc = closest obstacle on center ray

dl = closest obstacle on left   ray

dr = closest obstacle on right  ray

# 5. Finite State Machine  (CarFSM)

The FSM converts sensor readings into control commands (v, omega) using priority-ordered if/else rules.

## 5.1  Danger Threshold

th  =  sense_range * 0.52  =  2.0 * 0.52  =  1.04 m

## 5.2  State Transition Table

| Priority | Condition | New State | v (m/s) | omega (rad/s) |
| --- | --- | --- | --- | --- |
| 1 | dc < th | AVOID_L if dl >= dr AVOID_R if dl < dr | 1.5 | +1.6 or -1.6 |
| 2 | dl < th * 0.80 | AVOID_R | 1.5 | -1.6 |
| 3 | dr < th * 0.80 | AVOID_L | 1.5 | +1.6 |
| 4 | all clear | FORWARD | 2.5 | clip(2.2*herr, -1.6, +1.6) |
| - | d_goal < 0.55 m | ARRIVED | 0.0 | 0.0 |

## 5.3  Control Formula per State

FORWARD (path clear — proportional heading correction):

v = 2.5 m/s

omega = clip( 2.2 * herr,  -1.6,  +1.6 )  rad/s

AVOID_L (obstacle detected, left side has more space):

v = 1.5 m/s,    omega = +1.6 rad/s   (turn left)

AVOID_R (obstacle detected, right side has more space):

v = 1.5 m/s,    omega = -1.6 rad/s   (turn right)

ARRIVED (goal reached):

v = 0,   omega = 0

# 6. Simulation Loop  (run function)

Each iteration of the main for-loop executes the full sense-plan-act cycle:

| Step | Code | Formula / Description |
| --- | --- | --- |
| 1. Read state | get_state(data) | pos, yaw from sensors |
| 2. To goal | to_goal = goal - pos | d_goal = ¦¦to_goal¦¦ |
| 3. Heading err | herr = wrap(atan2-yaw) | Normalize to [-pi, pi] |
| 4. Goal check | if d_goal < 0.55 | -> ARRIVED, stop car |
| 5. Sensor scan | sensor.scan(pos, yaw) | Returns (dc, dl, dr) |
| 6. FSM decide | fsm.step(dc,dl,dr,herr) | Returns (v, omega) |
| 7. Apply vel | set_vel(data,model,v,omega) | Write to qvel joints |
| 8. Physics | mujoco.mj_step(model,data) | Advance by dt=0.01 s |
| 9. Log | logger.push(...) | Record all state data |
| 10. Render | viewer.sync() | Update 3D display |

## 6.1  Real-Time Pacing

step_time = 1 / sim_hz  =  1 / 40  =  0.025 s

sleep_time = step_time - elapsed_compute_time

sim_hz = 40 Hz  (from Config)

# 7. Full Parameter Reference

| Parameter | Value | Unit | Where Used |
| --- | --- | --- | --- |
| start | (-6.0, 0.0) | m | Car initial position |
| goal | (6.0, 0.0) | m | Target position |
| forward_vel | 2.5 | m/s | FORWARD state speed |
| turn_vel | 1.5 | m/s | AVOID_L / AVOID_R speed |
| turn_rate | 1.6 | rad/s | Omega in avoidance states |
| Kp (heading) | 2.2 | - | omega = clip(2.2*herr, ...) |
| dt | 0.01 | s | Physics timestep (XML) |
| sense_range | 2.0 | m | Max ray detection distance |
| sense_fov | 0.55 | rad | Side ray offset (~31.5 deg) |
| th (danger) | 1.04 | m | sense_range * 0.52 |
| goal_tol | 0.55 | m | Arrival radius |
| max_steps | 10,000 | steps | Simulation limit |
| sim_hz | 40 | Hz | Real-time display rate |

VGU  |  Programming Methods 2  |  MuJoCo Car Simulation  |  Rule-Based Obstacle Avoidance